Cairo University Faculty of Engineering

12th Undergraduate Engineering Mathematics Engineering Research Forum

Data Architects

Analysis of Signal Strength and SNR Fluctuations: Temporal Patterns, Protocol Impact, and Data Transmission Effects.

## Authors

Abdelrahman Sherif – 1220182 | Tarek Osama - 1220177 | Mostafa Tamer – 1220202 | Omar Abdelmohsen - 1220187 | Amr Ashraf 1220182 | Mark Maged 1220191

##### Under the supervision of:

Dr. Maha Amin Hassanein

MTHS204

Spring 2025

# Abstract

Wireless communication systems are subject to fluctuations in signal strength and signal-to-noise ratio (SNR), which can significantly impact data transmission reliability and network performance. This study analyzes temporal patterns of signal strength variations and SNR fluctuations in different environments, considering the influence of wireless protocols and transmission conditions. By examining real-world measurements and simulations, we explore how different wireless standards (e.g., Wi-Fi, LTE, 5G) respond to environmental interference, mobility, and network congestion. Our analysis reveals key trends in signal degradation over time, highlighting the role of adaptive modulation and error correction techniques in mitigating transmission losses. Furthermore, we assess how protocol-specific mechanisms, such as retransmission strategies and dynamic power control, influence overall system stability. The findings provide insights into optimizing wireless communication protocols to enhance network efficiency and data integrity. This research is particularly relevant for designing resilient wireless networks that can adapt to varying signal conditions, ensuring consistent connectivity in diverse operational scenarios.

# Table of Contents

Abstract	1

Table of Contents	2

List of Figures	3

Chapter 1: Problem Definition	4

  - Problem Description	4

  - Statistical Questions	4

Chapter 2: Descriptive Statistics	5

  - Definition of Variables	5

  - Data Description:	5

2.2.1 Dataset 1: 	5

2.2.2 Dataset 2: 	9

2.2.3 Dataset 3: 	14

Chapter 3: Methodology	10

  - Data Description	10

    - Description of Images	11

    - Descriptive Statistical Analyses	11

  - Model Architecture and Training Procedure	14

    - Layers Parameters	14

    - Model Training	15

  - Results	15

Conclusion	17

References	18

Appendix	19

# List of Figures

Figure 1 Dataset 1 Descriptive Statistics Table	5

Figure 2 Dataset 1 Heatmap	6

Figure 3 Distribution of Flow Duration	6

Figure 4 Distribution of Total Length of Forward Packets	7

Figure 5 Distribution of Total Length of Backward Packets	7

Figure 6 Distribution of Forward IAT	8

Figure 7 Distribution of Backward IAT	8

Figure 8 Distribution of Protocols	9

Figure 9 Dataset 2 Descriptive Statistics Table	9

Figure 10 Dataset 2 Heatmap	10

Figure 11 Distribution of Tx	10

Figure 12 Distribution of Rx	11

Figure 13 Distribution of SNR Receiver	11

Figure 14 Distribution of BER Receiver	12

Figure 15 Distribution of Transmission Distance	12

Figure 16 Distribution of Fiber Attenuation	13

Figure 17 Distribution of Temperature	13

Figure 18 Distribution of Humidity	14

Figure 19 Dataset 3 Descriptive Statistics Table	14

Figure 20 Dataset 3 Heatmap	15

Figure 21 Distribution of Signal Strength	15

Figure 22 Distribution of SNR	16

Figure 23 Distribution of Call Duration	16

Figure 24 Distribution of Attenuation	17

Figure 25 Distribution of Distance to Tower	17

Figure 26 Distribution of Environment	18

Figure 27 Distribution of Incoming/Outgoing	18

Figure 27 Distribution of Call Type	19

# Chapter 1: Problem Definition

### Problem Description

Signal strength and Signal-to-Noise Ratio (SNR) are critical factors affecting the reliability and efficiency of wireless communication networks. These parameters fluctuate due to various factors, including time of day, protocol used, data type/size, and distance from transmission sources. However, the extent and patterns of these variations are not always well understood. This project aims to analyze how signal strength and SNR change over time, their correlation with different communication protocols, and the impact of data characteristics. The findings will help identify potential optimization strategies for improving network performance.

1.2 Statistical Questions

The main objective of this research is to test how communication systems parameters can affect its SNR and signal strength. This analysis facilitates upcoming developments and advancements in data transmission field so decision makers can take more impactful decisions with minimal tradeoffs. We concluded it into 5-6 main statistical questions we ought to answer.

- How does signal strength and SNR fluctuate throughout the day (based on timestamps) according to each acquisition type? Are there peak hours of degradation?

- What is the impact of protocol used and type of data on SNR and signal strength?

- How is the Performance of protocols on different types/sizes of data throughout the day/year?

- Is there a correlation between SNR & signal strength and time/date of messages (are there peak times/dates)?

- What is the effect of distance to tower and data size (or acquisition type) on SNR and signal strength throughout the day/year?

- What is the effect of environmental conditions with different transmission distances on network performance?

# CHAPTER 2: DESCRIPTIVE STASTISTICS

### 2.1 Definition of Variables

Independent Variables: Timestamp, Total.Length.of.Fwd.Packets, Total.Length.of.Bwd.Packets, L7Protocol, ProtocolName, Transmission Distance, Temperature, Humidity, Call Duration (s), Environment, Distance to Tower (km), Call Type, Incoming/Outgoing.

Dependent Variables: Flow.Duration, Fwd.IAT.Total, Bwd.IAT.Total, Transmitter Power Level (Tx), Receiver Power Level (Rx), SNR Receiver, BER Receiver, Fiber Attenuation, Signal Quality, Signal Strength (dBm), SNR,  Attenuation.

### 2.2 Data Description

### 2.2.1 Dataset 1 (to answer Question 3): 8 Columns & 928,676 Rows

Fig.1 Dataset 1 Descriptive Statistics Table

Fig.2 Dataset 1 Heatmap

Fig.3 Distribution of Flow Duration

Fig.4 Distribution of Total Length of Forward Packets

Fig.5 Distribution of Total Length of Backward Packets

Fig.6 Distribution of Forward IAT

Fig.7 Distribution of Backward IAT

Fig.8 Protocols Distribution

### 2.2.2 Dataset 2 (to answer Question ): 9 Columns & 586 Rows

Fig.9 Descriptive Statistics Table

Fig.10 Dataset 2 Heatmap

Fig.11 Distribution of Tx

Fig.12 Distribution of Rx

Fig.13 Distribution of SNR Receiver

Fig.14 Distribution of BER Receiver

Fig.15 Distribution of Transmission Distance

Fig.16 Distribution of Fiber Attenuation

Fig.17 Distribution of Temperature

Fig.18 Distribution of Humidity

### 2.2.3 Dataset 3 (to answer Question ): 9 Columns & 463 Rows

Fig.19 Dataset 3 Descriptive Statistics Table

Fig.20 Dataset 3 Heatmap

Fig.21 Distribution of Signal Strength

Fig.22 Distribution of SNR

Fig.23 Distribution of Call Duration

Fig.24 Distribution of Attenuation

Fig.25 Distribution of Distance to Tower

Fig.26 Distribution of Environment

Fig.27 Distribution of Incoming/Outgoing

Fig.28 Distribution of Call Type

# Chapter 3: Methodology

The methodology is divided into three main parts: the Artificial Intelligence (AI) model, image preprocessing, and desktop application. The following sections delve deeply into each of these components.

2. Research Question: What is the impact of call duration and type of data on SNR and signal strength?

After collecting data from the schemas here is what we found:

Call duration:

Although the effect of call duration is not significant, the SNR tends to increase with the increase of call duration. (this might be due to the fact that as the time increases the probability of noise happening due to other reasons than the call duration itself)

Type of data:

The type of data did not make much of a difference as average SNR for data and voice consecutively is 20.31 and 19.33

Analysis of Variance (ANOVA): A one-way ANOVA test was performed to assess how SNR varies across call length and type of data (data, voice).

Ho:

- Call Duration: There is no significant relationship between call duration and SNR. Any observed increase in SNR with longer call durations is due to random variation or external factors unrelated to the call duration itself.

- Data Type: There is no significant difference in SNR between voice and data calls. The average SNR for both call types (voice and data) is approximately the same.

Ha:

- Call Duration: There is a significant relationship between call duration and SNR. Longer call durations lead to a noticeable increase in SNR.

- Data Type: There is a significant difference in SNR between voice and data calls. One type of data (voice or data) exhibits a higher or lower average SNR compared to the other.

Hypothesis Testing: test were made to test whether the call duration or data type has an effect on the SNR. The hypothesis tested was: "Data type almost has no effect on the SNR, and the call duration has a small effect on the SNR."

# Conclusion

pllm k

# References

- Nathaniel Handan. “OptiCom Signal Quality Dataset” https://www.kaggle.com/datasets/tinnyrobot/opticom-signal-quality-dataset?select=ocrdataset.csv Feb.15,2024 [March.7,2025]

- Asfand Yar. “Internet Traffic Data Set”

https://www.kaggle.com/datasets/asfandyar250/network/data  Feb.23,2023 [March.7,2025].

- Suraj “cellular-network-performance-data”

https://www.kaggle.com/datasets/suraj520/cellular-network-performance-data

Jan.01,2023 [March.7,2025].

# Appendix

Python code to process the data and apply the explained descriptive statistics analysis

import matplotlib.pyplot as plt
import seaborn as sns
import pandas as pd
import numpy as np
import pandas as pd

# Load dataset
df = pd.read_csv("C:/Users/osama/OneDrive/Desktop/Data_Architects/Datasets/Dataset-Unicauca-Version2-87Atts.csv")


# Drop "L7Protocol" column if it exists
if "L7Protocol" in df.columns:
    df = df.drop(columns=["L7Protocol"])

# Get descriptive statistics (excluding "count" row)
desc_stats = df.describe().drop(index="count", errors="ignore")

# Convert values to 2 decimal places (removes scientific notation)
desc_stats = desc_stats.applymap("{:.2f}".format)

# Convert DataFrame to a string format for visualization
fig, ax = plt.subplots(figsize=(12, 6))  # Set figure size
ax.axis('off')  # Hide axes
table = ax.table(cellText=desc_stats.values,
                 colLabels=desc_stats.columns,
                 rowLabels=desc_stats.index,
                 cellLoc='center', loc='center')

# Set table style
table.auto_set_font_size(False)
table.set_fontsize(10)
table.auto_set_column_width([i for i in range(len(desc_stats.columns))])

plt.show()

# Plotting correlation heatmap
plt.figure(figsize=(10, 6))
#correlation_matrix = df.corr()
correlation_matrix = df.select_dtypes(include=['number']).corr()

sns.heatmap(correlation_matrix, annot=True, cmap='coolwarm', fmt='.2f', linewidths=0.5)
plt.title('Correlation Heatmap')
plt.show()


# Convert relevant columns to numeric
numerical_columns = ['Flow.Duration', 'Total.Length.of.Fwd.Packets',
                     'Total.Length.of.Bwd.Packets', 'Fwd.IAT.Total', 'Bwd.IAT.Total']

# Define numerical columns
numerical_columns = ['Flow.Duration', 'Total.Length.of.Fwd.Packets',
                     'Total.Length.of.Bwd.Packets', 'Fwd.IAT.Total', 'Bwd.IAT.Total']

# Convert to numeric and clean data
for col in numerical_columns:
    df[col] = pd.to_numeric(df[col], errors='coerce')  # Convert to numeric
    df[col] = df[col].replace([np.inf, -np.inf], np.nan)  # Remove infinities

# Convert Flow Duration to seconds (assuming it's in microseconds)
df['Flow.Duration'] = df['Flow.Duration'] / 1_000_000
df['Fwd.IAT.Total'] = df['Fwd.IAT.Total'] / 1_000_000
df['Bwd.IAT.Total'] = df['Bwd.IAT.Total'] / 1_000_000

# Plot each numerical column separately
for col in numerical_columns:
    plt.figure(figsize=(8, 5))
    sns.histplot(df[col].dropna(), bins=50, color='skyblue')  # More bins for better detail
    plt.xlabel(f"{col} ({'10^6 seconds' if col in ['Flow.Duration', 'Fwd.IAT.Total', 'Bwd.IAT.Total'] else 'units'})")
    plt.ylabel("Count")
    plt.title(f"Distribution of {col}")
    plt.ticklabel_format(style='plain')  # Remove scientific notation
    plt.grid(True)
    plt.show()  #show each plot separately

# Bar plot for ProtocolName (categorical data)
plt.figure(figsize=(10, 6))
sns.countplot(y='ProtocolName', data=df, palette='Set2')
plt.title('Protocol Name Count Distribution')
plt.xlabel('Count')
plt.ylabel('Protocol Name')
plt.show()

# Load dataset

df = pd.read_csv("C:/Users/osama/OneDrive/Desktop/Data_Architects/Datasets/ocrdataset.csv")

# Generate descriptive statistics

desc_stats = df.describe().round(2)  # Round for better readability

# Create the figure

fig, ax = plt.subplots(figsize=(12, 6))  # Larger size for clarity

ax.axis('off')  # Hide axes

# Create the table with better styling

table = ax.table(cellText=desc_stats.values,

colLabels=desc_stats.columns,

rowLabels=desc_stats.index,

cellLoc='center',

loc='center',

colColours=['lightgray'] * desc_stats.shape[1])  # Header row shading

table.auto_set_font_size(False)

table.set_fontsize(10)  # Increase font size

table.auto_set_column_width([i for i in range(len(desc_stats.columns))])  # Auto-adjust width

# Add title

plt.title("Descriptive Statistics", fontsize=14, fontweight="bold", pad=20)

plt.show()

# Correlation Heatmap

# Convert "Good" to 1 and "Bad" to 0 (Assuming the column name is 'Label')

if 'Label' in df.columns:

df['Label'] = df['Label'].map({'Good': 1, 'Bad': 0})

# Ensure all columns are numeric for correlation

df_numeric = df.select_dtypes(include=[np.number])

# Plot the heatmap

plt.figure(figsize=(10, 6))

correlation_matrix = df_numeric.corr()

sns.heatmap(correlation_matrix, annot=True, cmap='coolwarm', fmt=".2f", linewidths=0.5)

plt.title("Correlation Heatmap")

plt.show()

# Plot distributions for numerical columns

numerical_columns = df.select_dtypes(include=[np.number]).columns.tolist()

for col in numerical_columns:

plt.figure(figsize=(8, 5))

sns.histplot(df[col].dropna(), bins=50, color='skyblue')  # Drop NaNs for cleaner plot

plt.xlabel(f"{col}")

plt.ylabel("Count")

plt.title(f"Distribution of {col}")

plt.ticklabel_format(style='plain')  # Avoid scientific notation

plt.grid(True)

plt.show()

# Load dataset
df = pd.read_csv("C:/Users/osama/OneDrive/Desktop/Data_Architects/Datasets/train.csv")

# Generate descriptive statistics
desc_stats = df.describe().round(2)
# Save table as an image
fig, ax = plt.subplots(figsize=(12, 6))
ax.axis('tight')
ax.axis('off')
table = ax.table(cellText=desc_stats.values, 
                 colLabels=desc_stats.columns, 
                 rowLabels=desc_stats.index, 
                 cellLoc='center', 
                 loc='center',
                 colColours=['lightgray'] * desc_stats.shape[1])  

table.auto_set_font_size(False)
table.set_fontsize(10)
table.auto_set_column_width([i for i in range(len(desc_stats.columns))])  

plt.title("Descriptive Statistics", fontsize=14, fontweight="bold", pad=20) 
plt.show()


df_numeric = df.select_dtypes(include=[np.number])

# Correlation Heatmap (Numeric Only)
plt.figure(figsize=(10, 6))
correlation_matrix = df_numeric.corr()
sns.heatmap(correlation_matrix, annot=True, cmap='coolwarm', fmt=".2f", linewidths=0.5)
plt.title("Correlation Heatmap")
plt.show()

# Plot distributions for each numerical column
for col in df_numeric.columns:
    plt.figure(figsize=(8, 5))
    sns.histplot(df[col].dropna(), bins=50, color='skyblue')
    plt.xlabel(f"{col}")
    plt.ylabel("Count")
    plt.title(f"Distribution of {col}")
    plt.grid(True)
    plt.show()

# Identify categorical columns
categorical_columns = [col for col in df.select_dtypes(include=['object']).columns
                       if 'time' not in col.lower() and 'date' not in col.lower()]

# Plot bar charts for each categorical column
for col in categorical_columns:
    plt.figure(figsize=(8, 5))
    sns.countplot(y=df[col], palette='Set2', order=df[col].value_counts().index)
    plt.xlabel("Count")
    plt.ylabel(col)
    plt.title(f"Distribution of {col}")
    plt.show()

Dataset used:

https://github.com/TarekOsama528/Data_Architects/tree/main/Datasets